Document Title: Product Requirements Document (PRD) Document ID: HS-REQ-001 Version: 1.0 Status: DRAFT Author: Sandhya Patel Date: September 8, 2025

- Introduction

This document defines the product requirements for the HydroSync system. All requirements listed herein are mandatory for the product's release. Each requirement must be verified according to its specified verification method. All requirements are traceable to the System Use Definition document (HS-SUD-001).

2.0 Requirements

| ID | Requirement Description | Verification Method | Traceability |
| --- | --- | --- | --- |
| Section 2.1: Hardware Requirements |  |  |  |
| REQ-001 | The wearable device shall incorporate a Galvanic Skin Response (GSR) sensor to measure skin conductivity. | Test, Inspection | HS-SUD-001 |
| REQ-002 | The wearable device shall incorporate a pulse sensor capable of deriving Heart Rate Variability (HRV). | Test, Inspection | HS-SUD-001 |
| REQ-003 | The wearable device shall incorporate a temperature sensor to measure peripheral skin temperature. | Test, Inspection | HS-SUD-001 |
| REQ-004 | The wearable device shall use an ESP32 microcontroller for all data processing and control functions. | Inspection | HS-SUD-001 |
| REQ-005 | The wearable device shall be powered by a rechargeable Lithium-Ion battery. | Inspection | HS-SUD-001 |
| Section 2.2: Firmware Requirements |  |  |  |
| REQ-006 | The firmware shall acquire data from all integrated sensors (GSR, HRV, Temp) at a user-configurable sync interval. | Test, Analysis | HS-SUD-001 |
| REQ-007 | The firmware shall transmit the acquired sensor data to the paired mobile application using Bluetooth Low Energy (BLE). | Test | HS-SUD-001 |
| Section 2.3: Mobile Application Requirements |  |  |  |
| 2.3.1 Dashboard |  |  |  |
| REQ-008 | The mobile application shall be capable of establishing and maintaining a BLE connection with the wearable device. | Test | HS-SUD-001 |
| REQ-009 | The mobile application dashboard shall display the user's key physiological metrics, including a hydration metric (derived from GSR), a fatigue/stress metric (derived from HRV), and skin temperature. | Test, Inspection | HS-SUD-001 |
| REQ-010 | The mobile application dashboard shall display a primary status banner (e.g., "Optimal," "Fatigued," "Dehydrated"). | Test, Inspection | HS-SUD-001 |
| REQ-011 | The color of the primary status banner shall change to indicate the user's state (e.g., green for 'Optimal', red for 'Fatigued'). | Test, Inspection | HS-SUD-001 |
| REQ-012 | The mobile application dashboard shall provide navigation buttons to "View Trends," "View Alerts," and "User Profile." | Test, Inspection | HS-SUD-001 |
| 2.3.2 Trends Screen |  |  |  |
| REQ-013 | The Trends screen shall display a graphical plot of the user's historical physiological data over time. | Test, Inspection | HS-SUD-001 |
| REQ-014 | The Trends screen shall provide filter options to view data for individual metrics (e.g., Hydration, Fatigue, Temperature). | Test | HS-SUD-001 |
| 2.3.3 Alerts Screen |  |  |  |
| REQ-015 | The Alerts screen shall display a list of all historical alert events. | Test, Inspection | HS-SUD-001 |
| REQ-016 | Each alert in the list shall include a description (e.g., "Fatigue Detected"), the measured value that triggered it, and a timestamp. | Test, Inspection | HS-SUD-001 |
| REQ-017 | The Alerts screen shall provide a function for the user to dismiss or clear alerts from the list. | Test | HS-SUD-001 |
| REQ-018 | The Alerts screen shall provide a function to export the alert history as a CSV file. | Test | HS-SUD-001 |
| 2.3.4 User Profile & Recommendations Screen |  |  |  |
| REQ-019 | The app shall include a User Profile screen that displays user information and provides behavioral recommendations. | Test, Inspection | HS-SUD-001 |
| REQ-020 | The Recommendations section shall provide actionable suggestions based on the user's current physiological state (e.g., "Hydration is low. Drink water."). | Test | HS-SUD-001 |
| REQ-021 | The User Profile screen shall provide a function to un-pair the wearable device or log out of the application. | Test | HS-SUD-001 |
| 2.3.5 Settings Screen |  |  |  |
| REQ-022 | The Settings screen shall allow the user to configure the alert thresholds for hydration, fatigue, and temperature. | Test | HS-SUD-001 |
| REQ-023 | The Settings screen shall allow the user to select the data synchronization interval between the wearable and the app (e.g., 5s, 10s, 30s, 60s). | Test | HS-SUD-001 |
| REQ-024 | The Settings screen shall allow the user to select a "User Type" profile (e.g., Student, Athlete) to potentially adjust the system's algorithms. | Test, Inspection | HS-SUD-001 |